TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

All elements of this unit are chemically resistant.

These stirred reactors in series are used to increase the reagents conversion referred to an only reactor and so obtain product with higher purity.

Three continuous stirred tank reactors connected in series.

The three reactors have different height to let product from the first reactor  go to the second one and so on.

Reactors body made of pyrex glass, volume: 2 l.

Each reactor is fitted with a conductivity cell. Measurement range up to 20 mS.

Each reactor has a stirrer.

The two reagent vessels and the two variable speed dosing pumps (at the Service Unit for QR, “QUS”) feed reagents into the first reactor in line.

A dead-time residence coil can also be attached to the exit of the last reactor in the series.

Three temperature sensors “J” type, one in each reactor.

Quick connectors with shutoff valve that enable an easy coupling of the reactor to the Service Unit for QR, “QUS”.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals..

Dimensions: 950 x 450 x 600 mm approx. (37.40 x 17.71 x 23.62 inches approx.). Weight: 35 kg approx. (77 pounds approx.)

Required elements (Not included):

QUS. Service Unit for QR.

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This unit is common for the required elements (at least one) (Not included).

Accommodation and exchange system of the reactors, quick and easy to handle.

It supplies all the services for the operation of each reactor.

Safety, easy and quick connections.

All elements of this unit are chemically resistant.

The “QUS” unit mainly consists of:

Two peristaltic dosing pumps, with variable speed. Flow rate up to 3  l/h (unit standard disposition). With another disposition, they could reach a flow rate up to 10  l/h.

Thermostatic bath, capacity: 6  l. Temperature control of hot water in order to maintain the reactor temperature.

Pump of 3  l/min, to impel the thermostatization water from the bath to the reactor.

Two tanks for the reagents, capacity: 1 l each one, made of Pyrex glass.

The control of the reaction is carried out by a conductivity sensor, which allows the reaction evolution parameterization in real time.

Three “J” type temperature sensors, one to know the thermostatic bath temperature in a continuous way and two sensors to know the water temperature of the thermostatic bath water inlet and outlet.

Quick connectors with shutoff valve that enable an easy coupling of the Service Unit to the chosen reactor.

Electronic console:

Metallic box.

Temperature sensors connections.

Digital display for temperature sensors.

Selector for temperature sensors.

Peristaltic pumps controllers and switches.

Water pump switch.

Stirrer switches.

Heating element controller.

Main switch.

EXERCISES AND PRACTICAL POSSIBILITIES

1.-Investigation of dynamic behaviour of stirred tank reactors in series.

2.-Determination of the ionic conductivities.

3.-Influence of flow rate.

4.-Work with just one reactor in continuous.

5.-Work with just one reactor in continuous with mixture effects.

6.-Work with three reactors in continuous.

7.-Effect of step input change.

8.-Response to an impulse change.

9.-Investigation of time constant using dead time coil.

Optional

QRS/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.