TENDER SPECIFICATIONS (for main items)

1 QUSC. Service Unit for QRC:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This unit is common for the required elements (at least one) (Not included).

Accommodation and exchange system of the reactors, quick and easy to handle.

It supplies all the services for the operation of each reactor.

Safety, easy and quick connections.

All elements of this unit are chemically resistant.

The “QUSC” unit mainly consists of:

Two computer controlled peristaltic dosing pumps, with variable speed. Flow rate up to 3  l/h (unit standard disposition). With another disposition, they could reach a flow rate up to 10 l/h.

Computer controlled thermostatic bath, capacity: 6  l. Temperature PID control of the thermostatic bath.

Pump of 3 l/min, to impel the thermostatization water from the bath to the reactor.

Flow sensor, range: 0.25 – 6.5 l/min.

Two tanks for the reagents, capacity: 1 l each one, made of Pyrex glass.

The control of the reaction is carried out by a conductivity sensor, which allows the reaction evolution parameterization in real time.

Three “J” type temperature sensors, one to know the thermostatic bath temperature in a continuous way and two sensors to know the water temperature of the thermostatic bath water inlet and outlet.

Quick connectors with shutoff valve that enable an easy coupling of the Service Unit to the chosen reactor.

The complete unit includes as well:

Advanced Real-Time SCADA and PID Control.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

National Instruments Data Acquisition board (250 KS/s, kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc. Apart from monitoring user's knowledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

Required elements (at least one) (Not included):

QRCAC. Computer Controlled Continuous Stirred Tank Reactor for QRC.

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

All elements of this unit are chemically resistant.

Designed to demonstrate the  behaviour of a reactor used for homogeneous reactions liquid-liquid.

Reactor body made of borosilicate glass, with a maximum capacity of 2 l, specially designed to work in continuous. It also allows batch operation.

Adjustable volume, range: 0.4 – 1.5 l.

Stainless steel heat transfer coil (5 loops of 60 mm of diameter) and a baffle (removable).

Computer controlled stirring system, with speed control and indication. Stirrer range: 0 – 220 rpm.

Reactor lip with connectors for the appropriate sensors.

Temperature sensor “J” type to control the temperature into the reactor.

Conductivity cell to control the reaction. Measurement range up to 20 mS.

Quick connectors with shutoff valve that enable an easy coupling of the reactor to the Service Unit for QRC, “QUSC”.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

QRCAC/CCSOF. PID Computer Control + Data Acquisition + Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Analog and digital PID control.

PID menu and set point selection required in the whole work range.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

QRTC. Computer Controlled Tubular Flow Reactor for QRC.

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

All elements of this unit are chemically resistant.

Reactor composed by a continuous tube where the reagents are introduced through the coil end and  the products are obtained through the inverse end. Into it, a continuous reagent mix is produced, so  the composition will be different at each point. This type of reactors are industrially used for  homogeneous reactions liquid-liquid, generally in isothermal conditions.

With this small computer controlled scale reactor, the behaviour of this type of reactors used at industrial level can be observed.

Tubular flow reactor, volume: 0.4  l. Coil shaped. Placed into an acrylic vessel through which the cooling or heating medium circulates. Coil length: 20 m.

Electric pre-heater of 12 loops, and loop diameter of 70 mm approx, for the two reagents feed lines.

It is placed before the mix and the currents inlet to the reactor.

Temperature controlled by water jacketed.

Two temperature sensors “J” type to know the reagents outlet temperature from the pre-heater.

Conductivity cell to control the reaction. Measurement range up to 20 mS.

Reactor lip with connectors for the appropriate sensors.

Quick connectors with shutoff valve that enable an easy coupling of the reactor to the Service Unit for QRC, “QUSC”.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

QRTC/CCSOF. PID Computer Control + Data Acquisition + Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Analog and digital PID control.

PID menu and set point selection required in the whole work range.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

QRDC. Computer Controlled Batch Reactor for QRC.

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

All elements of this unit are chemically resistant.

Designed for the kinetic study of homogeneous reactions liquid-liquid, both in adiabatic conditions and in isothermal conditions.

The reactor body is an isolated vessel with a stainless steel external casing, working volume: 1 l.

Heat transfer coil made of stainless steel and reactor of 5 loops of 60 mm of diameter. The baffle, tube internal diameter is of 6 mm and the external one is of 8 mm.

Computer controlled stirring system, with speed control and indication. Stirrer range: 0 – 220 rpm.

Temperature sensor “J” type to control the temperature into the reactor.

Conductivity cell to control the reaction. Measurement range up to 20 mS.

Reactor lip with connectors for the appropriate sensors.

Quick connectors with shutoff valve that enable an easy coupling of the reactor to the Service Unit for QRC, “QUSC”.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

QRDC/CCSOF. PID Computer Control + Data Acquisition + Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Analog and digital PID control.

PID menu and set point selection required in the whole work range.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

QRSC. Computer Controlled Stirred Tank Reactors in Series for QRC.

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

All elements of this unit are chemically resistant.

These stirred reactors in series are used to increase the reagents conversion referred to an only reactor and so obtain product with higher purity.

Three computer controlled continuous stirred tank reactors connected in series.

The three reactors have different height to let product from the first reactor  go to the second one and so on.

Reactors body made of pyrex glass, volume: 2 l. Adjustable volume, range: 0.4 – 1.5 l.

Each reactor is fitted with a conductivity cell. Measurement range up to 20 mS.

Each reactor has a computer controlled stirrer with variable speed.

The two reagent vessels and the two variable speed dosing pumps (at the Service Unit for QRC, “QUSC”) feed reagents into the first reactor in line.

A dead-time residence coil can also be attached to the exit of the last reactor in the series.

Three temperature sensors “J” type, one in each reactor.

Quick connectors with shutoff valve that enable an easy coupling of the reactor to the Service Unit for QRC, “QUSC”.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

QRSC/CCSOF. PID Computer Control + Data Acquisition + Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Analog and digital PID control.

PID menu and set point selection required in the whole work range.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

QRLC. Computer Controlled Laminar Flow Reactor for QRC.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

All elements of this unit are chemically resistant.

Designed to demonstrate the flow pattern characterisation and the steady state conversion in a tubular reactor.

Working volume: 400 ml.

Laminar flow reactor constituted by a glass column of 400 ml and 1000 mm long, including two diffusers packed with glass balls of 3 mm.

At the bottom of the column a premixer provides a complete mixing of the reagents entering the reactor and improves the flow distribution.

The reactor refrigeration jacket keeps its contents at a constant temperature to keep the laminar flow conditions.

Temperature sensor “J” type.

Conductivity cell to control the reaction. Measurement range up to 20 mS.

Quick connectors with shutoff valve that enable an easy coupling of the reactor to the Service Unit for QRC, “QUSC”.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

QRLC/CCSOF. PID Computer Control + Data Acquisition + Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Analog and digital PID control.

PID menu and set point selection required in the whole work range.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

QRPC. Computer Controlled Plug Flow Reactor for QRC.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

All elements of this unit are chemically resistant.

Designed to demonstrate the flow pattern characterisation and the steady state conversion in a tubular reactor with axial dispersion.

Working volume: 1 l.

Plug reactor constituted by a glass column of 1 l and 1100 mm long, packed with 3 mm diameter glass balls.

At the bottom of the column a premixer provides a complete mixing of the reagents entering the reactor and improves the flow distribution.

The unit uses a six ways injection valve, which allows either the feeding of reagents in a continuous way or the possibility to carry out pulse and step changes to characterization of the flow pattern.

Temperature sensor “J” type.

Conductivity cell to control the reaction. Measurement range up to 20 mS.

Quick connectors with shutoff valve that enable an easy coupling of the reactor to the Service Unit for QRC, “QUSC”.

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

QRPC/CCSOF. PID Computer Control + Data Acquisition + Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Analog and digital PID control.

PID menu and set point selection required in the whole work range.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

2 QRC/CIB. Control Interface Box:

This control interface is common for the required elements (at least one) (Not included).

Control interface box with process diagram in the front panel.

The unit control elements are permanently computer controlled.

Simultaneous visualization in the computer of all parameters involved in the process.

Calibration of all sensors involved in the process.

Real time curves representation about system responses.

All the actuators’ values can be changed at any time from the keyboard allowing the analysis about curves and responses of the whole process.

Shield and filtered signals to avoid external interferences.

Real time PID control with flexibility of modifications from the computer keyboard of the PID parameters, at any moment during the process.

Real time PID control for parameters involved in the process simultaneously.

Proportional control, integral control and derivative control, based on the real PID mathematical formula, by changing the values, at any time, of the three control constants (proportional, integral and derivative constants).

Open control allowing modifications, at any moment and in real time, of parameters involved in the process simultaneously.

Three safety levels, one mechanical in the unit, another electronic in the control interface and the third one in the control software.

3 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

The Data Acquisition board model may change at any moment, providing the same or better features than those required for the unit.

4 Cables and Accessories, for normal operation.

5 Manuals:

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

Practices to be done with the Computer Controlled Continuous Stirred Tank Reactor for QRC (QRCAC):

1.-  Determination of the ionic conductivities.

2.- Batch operation. Obtaining of the reaction order respect to ethylacetate. Initial velocity method.

3.- Batch operation. Obtaining of the reaction order respect to sodium hydroxide. Initial velocity method.

4.- Batch operation. Velocity Constant Computation. Constant sodium hydroxide initial concentration.

5.- Batch operation. Velocity Constant Computation. Constant ethylacetate initial concentration.

6.-  Velocity equation formulation.

7.-   Batch operation. Variation of the kinetic constant with temperature. Arrhenius Equation.

8.- Batch operation. Theoretical and experimental conversion comparative. Deviation from ideality.

9.-   Batch operation. Mixture effects.

10.- Continuous operation.

11.- Continuous operation. Mixture effects.

12.- Conductivity measurement system: conductimeter.

13.- Variation of conversion with residence time.

14.- Residence time distribution.

15.- Determination of the reaction rate constant.

Additional practical possibilities:

16.- Sensors calibration.

Practices to be done with the Computer Controlled Tubular Flow Reactor for QRC (QRTC):

17.- Analysis of reagents and products.

18.- Ionic conductivities determination.

19.- Theoretical conversion of the tubular reactor.

20.- Experimental determination of the conversion of the tubular reactor.

21.- Dependence of the residence time.

22.- Determination of the reaction order.

23.- Dependence of the speed constant and the conversion with the temperature.

24.- Conductivity measurement system: conductimeter.

25.- Complete emptying of the unit.

26.- Determination of the reaction rate constant.

Additional practical possibilities:

27.- Sensors calibration.

Practices to be done with the Computer Controlled Batch Reactor for QRC (QRDC):

28.-Determination of the ionic conductivities.

29.-Batch operation. Calculation of the order of the reaction referred to the ethyl-acetate. Initial velocity method.

30.-Batch operation. Determination of the order of the reaction referred to the sodium hydroxide. Initial velocity method.

31.-Batch operation. Determination of the speed constant, the initial concentration of the sodium hydroxide is constant.

32.-Batch operation. Determination of the speed constant, the initial concentration of the ethyl acetate is constant.

33.-Formulation of the speed equation.

34.-Batch operation. Variation of the kinetic constant when the temperature is not constant: Arrhenius equation.

35.-Batch operation. Comparison of the theoretical and the experimental conversion: Deviation from the ideality.

36.-Calculation of the heat transference coefficient of the coil.

37.-Calculation of the hydrolysis reaction enthalpy.

38.-Batch operation. Mixture effects.

39.-Conductivity measurement system: conductimeter.

Additional practical possibilities:

40.-Sensors calibration.

Practices to be done with the Computer Controlled Stirred Tank Reactors in Series for QRC (QRSC):

41.-Investigation of dynamic behaviour of stirred tank reactors in series.

42.-Determination of the ionic conductivities.

43.-Influence of flow rate.

44.-Work with just one reactor in continuous.

45.-Work with just one reactor in continuous with mixture effects.

46.-Work with three reactors in continuous.

47.-Effect of step input change.

48.-Response to an impulse change.

49.-Investigation of time constant using dead time coil.

Additional practical possibilities:

50.-Sensors calibration.

Practices to be done with the Computer Controlled Laminar Flow Reactor for QRC (QRLC):

51.-Determination of the residence time distribution of the reactor.

52.-Effect of flow rate and feed concentration on the determination of flow pattern.

53.-Steady state conversion for a reaction with laminar flow.

54.-Effect of flow rate and feed concentration on the steady state conversion.

55.-Demonstration of the flow pattern in the reactor and comparison with the theoretical model.

56.-Effect of the temperature on the laminar flow characterisation.

57.-Determination of the steady state conversion of a second order reaction.

58.-Flow pattern characterisation in a laminar flow reactor.

59.-Conductivity measurement system: conductimeter.

Additional practical possibilities:

60.-Sensors calibration.

Practices to be done with the Computer Controlled Plug Flow Reactor for QRC (QRPC):

61.-Determination of the residence time distribution of the reactor.

62.-Effect of flow rate and feed concentration on the determination of flow pattern.

63.-Study of the reactor response to different perturbations: step and pulse change.

64.-Effect of flow rate and feed concentration on the steady state conversion.

65.-Demonstration of the flow pattern in the reactor and comparison with the theoretical model.

66.-Determination of the steady state conversion of a second order reaction.

67.-Understanding the principles of tracer techniques in flow pattern characterisation.

68.-Conductivity measurement system: conductimeter.

Additional practical possibilities:

69.-Sensors calibration.

Other possibilities to be done with this Unit:

70.-Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

71.-Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivative parameters; etc, in real time.

72.-The Computer Control System with SCADA and PID Control allow a real industrial simulation.

73.-This unit is totally safe as uses mechanical, electrical/electronic, and software safety devices.

74.-This unit can be used for doing applied research.

75.-This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

76.-Control of the QUSC unit process through the control interface box without the computer.

77.-Visualization of all the sensors values used in the QUSC unit process.

- By using PLC-PI additional 19 more exercises can be done.

- Several other exercises can be done and designed by the user.

TENDER SPECIFICATIONS (for optional items)

a) Industrial configuration

6 PLC. Industrial Control using PLC (it includes PLC-PI Module plus PLC-SOF Control Software):

-PLC-PI. PLC Module:

Metallic box.

Circuit diagram in the module front panel.

Digital inputs (X) and Digital outputs (Y) block: 16 Digital inputs. 14 Digital outputs.

Analog inputs block: 16 Analog inputs.

Analog outputs block: 4 Analog outputs.

Touch screen.

Panasonic PLC:

High-speed scan of 0.32 µsec. Program capacity of 32 Ksteps. High-speed counter. Multi-point PID control.

Digital inputs/outputs and analog inputs/outputs Panasonic modules.

-QUSC/PLC-SOF. PLC Control Software:

Always included with PLC supply. Each chemical reactor has its own software.

Practices to be done with PLC-PI:

1.-  Control of the particular unit process through the control interface box without the computer.

2.-  Visualization of all the sensors values used in the particular unit process.

3.-  Calibration of all sensors included in the particular unit process.

4.-  Hand on of all the actuators involved in the particular unit process.

5.-  Realization of different experiments, in automatic way, without having in front the particular unit. (These experiments can be decided previously).

6.- 	Simulation of outside actions, in the cases do not exist hardware elements. (Example: test of complementary tanks, complementary industrialenvironment to the process to be studied, etc).

7.-  PLC hardware general use.

8.-  PLC process application for the particular unit.

9.-  PLC structure.

10.-PLC inputs and outputs configuration.

11.-PLC configuration possibilities.

12.-PLC program languages.

13.-PLC different programming standard languages (ladder diagram (LD), structured text (ST), instructions list (IL), sequential function chart (SFC), function block diagram (FBD)).

14.-New configuration and development of new process.

15.-Hand on an established process.

16.-To visualize and see the results and to make comparisons with the particular unit process.

17.-Possibility of creating new process in relation with the particular unit.

18.-PLC Programming Exercises.

19.-Own PLC applications in accordance with teacher and student requirements.

b) Technical and Vocational Education configuration

7 QUSC/ICAI. Interactive Computer Aided Instruction Software.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

8 QUSC/FSS. Faults Simulation System.

Faults Simulation System (FSS) is a Software package that simulates several faults in any EDIBON Computer Controlled Unit.

The "FAULTS" mode consists in causing several faults in the unit normal operation. The student must find them and solve them.

There are several kinds of faults that can be grouped in the following sections:

Faults affecting the sensors measurement:

- An incorrect calibration is applied to them.

- Non-linearity.

Faults affecting the actuators:

- Actuators channels interchange at any time during the program execution.

- Response reduction of an actuator.

Faults in the controls execution:

- Inversion of the performance in ON/OFF controls.

- Reduction or increase of the calculated total response.

- The action of some controls is annulled.

On/off faults:

- Several on/off faults can be included.

c) Multipost Expansions options

9 MINI ESN. EDIBON Mini Scada-Net System for being used with EDIBON Teaching Units.

MINI ESN. EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The MINI ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA and PID Control integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA and PID Control, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- MINI ESN Software to control the whole system.

- Cables and accessories required for a normal operation.