TENDER SPECIFICATIONS

The Transducers and Instrumentation Trainer “SAIT” shows didactically the function principles of the transducers most used in industry. It is divided into two parts: the lower part, in which all the input and output transducers are found, while in the upper part, the system of signal conditioning and those of instrumentation are found; the Electrical and Pneumatic power supplies are housed in its interior.

Input transducers:

Resistance Transducers for applications in angular or linear position:

Linearly sliding potentiometer.

Rotary carbon-track potentiometer.

Rotary coil potentiometer.

Precision servo-potentiometer.

The Wheatstone Bridge circuit.

Applications of temperature:

NTC (Negative Temperature Coefficient) Thermistors.

RTD Sensor (Platinum Transducer with Temperature dependent Resistance).

Temperature sensor IC “Integrated Circuit LM 335”.

Type “K” Thermocouples.

Applications of light:

Photovoltaic Cell.  Phototransistor.  Photodiode PIN.  Photoconductive Cell.

Linear position and force:

Linear Variable Differential Transformer LVDT.

Extensiometric Transducer.

Environmental measurements:

Air flow Sensor.  Air pressure Sensor.  Humidity sensor.

Rotational speed and position control:

Slotted optoelectronic Sensor.

Opto- reflective Sensor.

Inductive sensor.

Hall effect Sensor.

Permanent D.C. magnet tachogenerator.

Sound measurements:

Dynamical microphone.

Ultrasonic receiver.

Visualization devices:

Timing device/ counter with LED display.

Graphic bar visualizer.

Mobile coil voltmeter.

Output transducers:

Electrical Resistance.

Incandescent Lamp.

Applications for the sound output:

Buzzing (Buzzer).

Mobile coil loud speaker.

Ultrasonic transmitter.

Applications of linear or angular motion:

D.C. Soleinod.

D.C. Relay.

Solenoid Valve.

Permanent Magnet D.C. Motor.

Signal conditioners:

D.C. Amplifiers.  A.C. Amplifier.

Power Amplifier.  Current Amplifier.

Buffers.

Inverting Amplifier.

Differential amplifier.

V/F and F/V Converters.

V/I and I/V Converters.

Full Wave Rectifier.

Hysteresis convertible Comparator.

Electronic switch.

Oscillator 40 kHz.

Filter 40 kHz.

Time-constant convertible Low Pass Filter.

Circuit with Mathematical Operation:

Adding amplifier.

Integrator with different time constants. Differentiator with different time constants. Instrumentation Amplifier.

Circuit SAMPLE & HOLD.

Amplifiers with gain control and offset.

Furthermore it contains a linearly mounted system of a D.C. motor, tachodynamo, reflective, slotted opto-sensors to detect the absolute and incremental position.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Additional recommended elements (only one) (Not included):

- EDAS/VIS-0.25. EDIBON Data Acquisition System and Virtual Instrumentation (speed: 250,000 samples/s).

- EDAS/VIS-1.25. EDIBON Data Acquisition System and Virtual Instrumentation (speed: 1,250,000 samples/s).

1 EDAS/VIS. EDIBON Data Acquisition System and Virtual Instrumentation:

16 Analog inputs (1 block with 12 voltage channels and 1 block with 2 current channels (4 connections)).

EDAS/VIS-0.25, sampling velocity 250,000 samples per second.

EDAS/VIS-1.25, sampling velocity 1,250,000 samples per second.

2 analog outputs.

24 digital inputs/outputs, configurable as inputs or outputs, with 24 state LED indicators. These digital inputs/outputs are grouped in three ports of eight channels (P0, P1 and P3).

4 digital signal switches 0 - 5 V.

2 analog signal potentiometers 12 V.

Main ON/OFF switch.

Inside: internal power supply of 12 V and 5 V. Potentiometer.

2 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

For EDAS/VIS-1.25 Version EDIBON Data Acquisition System and Virtual Instrumentation (speed: 1,250,000 samples/s):

PCI Data acquisition board (National Instruments) to be placed in a computer slot. Bus PCI.

Analog input:

Number of channels=16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536.

Sampling rate up to: 1,250,000 S/s (samples per second). Input range (V)= ±10V.

Data transfers=DMA, interrupts, programmed I/0.  Number of DMA channels=6.

Analog output:

Number of channels=2. Resolution=16 bits, 1 in 65536. Max. output rate up to: 900 KS/s.

Output range(V)= ±10V. Data transfers=DMA, interrupts, programmed I/0.

Digital Input/Output: Number of channels=24 inputs/outputs. Port 0 up to 8 MHz.

Timing: Counter/timers=2. Resolution: Counter/timers: 32 bits.

For EDAS/VIS-0.25 Version EDIBON Data Acquisition System and Virtual Instrumentation (speed: 250,000 samples/s):

Sampling rate up to: 250,000 S/s (samples per second).

Analog output: Max. output rate up to: 10KS/s.

Digital Input/Output: Number of channels=24 inputs/outputs. Port 0 up 1 MHz.

Rest of characteristics are the same than EDAS/VIS 1.25 Version.

The Data Acquisition board model may change at any moment, providing the same or better features than those required for the unit.

3 EDAS/VIS/CCSOF. Computer Control + Data Acquisition + Data Management Software:

Compatible with actual Windows operating systems. Amicable graphical frame.

Configurable software allowing the temporal/frequency representation of the different inputs and outputs. Visualization of a voltage of the circuits on the computer screen.

It allows data store in a file, print screens and reports of the signals at any time.

Measurement, analysis, visualization, representation and report of results.

Set of virtual instruments:

Oscilloscope:

Channels: 12 simultaneous. Maximum input voltage: 10 V.

All 12 input channels could be scaled to compare signal with different voltage levels.

Maximum sampling velocity: 1000 samples per second.

“Math Menu” with operations as add, rest, multiplication and division, between any of the 12 oscilloscope channels.

Function generator:

Two independent signal generators, for sinusoidal, triangular, sawtooth and square.

Channels: 2 (allowing working simultaneously). Maximum output voltage: 10 V.

Maximum output rate: 1000 samples per second.

It includes a graph where an output signal for each channel is shown.

Spectrum analyzer:

Channels: 12 (simultaneous). Maximum voltage: 10 V. Digital spectrum analyzer: based on the FFT.

Maximum sampling velocity: 1000 samples per second.

Multimeter:

Voltmeter, channels: 12 (simultaneous). Maximum voltage: 10 V RMS.

Ammeter, channels: 2 (simultaneous). Maximum ampere: 500 mA RMS per channel.

Transient analyzer.

Logic analyzer:

Number of input channels: 8. TTL Voltage level.

Clock source: 2 different sources.

This instrument allows receiving as far as 8 digital signal simultaneously.

Logic generator:

Number of transmission channels: 8. TTL voltage level.

This instrument allows generating up to 8 digital simultaneous signals.

- Dimensions: 400 x 400 x 300 mm approx. - Weight: 10 kg approx.

EXERCISES AND PRACTICAL POSSIBILITIES

Some Practical Possibilities of the “SAIT” Trainer:

- Basic Control Systems description.

Characteristics of the Control System:

- Characteristics of an ON/OFF temperature control system.

- Characteristics of an ON/ OFF Lighting system.

- Investigation of the Characteristics of a Positional Control System.

- Proportional Control.

- Proportional+Integral Control.

- Proportional+Derivate Control.

- Proportional+Integral+Derivate Control.

- Characteristics of a Speed Control System.

- Operation in Open Loop.

- Operation in Closed Loop, Proportional Control.

- Proportional+Integral Control.

- Proportional+Integral+Derivate Control.

Display devices:

- Application of the Timer/Counter as a meter of time.

- Application of the Timer/Counter as a simple counter.

- Application of the Timer/Counter as rev-counter or frequency-meter.

- Characteristics of an L.E.D. bargraph display unit.

- Characteristic of a Mobile Coil Meter.

- Comparison of Digital, Bargraph and Mobile Coil meters.

- To widen the voltage index of the B. M. meter.

Variable Resistance transducers in angle or linear arrangement:

- Variation of the Output Voltage for a Potentiometer used as a Position transducer.

- The Buffer as compensator for the effect of the load on the output voltage of a potentiometer.

- Servo potentiometer. Variation of the output voltage with respect to its position.

- Measuring the Resistance using a Wheatstone Bridge Circuit.

- Measuring the Voltage using “Null Balance” Procedures (Method 1).

- Measuring Voltages using “Null Balance” Procedures (Method 2). Measuring voltages smaller than the normal available voltage.

- Measuring Voltages using “Null Balance” Procedures (Method 2). Measuring voltages greater than the normal voltage.

Transducers for Applications of Temperature Measurement:

- Characteristics of an Integrated Temperature Circuit.

- Construction of a Digital Thermometer using the facilities of the TRANSDUCER TRAINER.

- Characteristics of a Platinum Temperature Dependent Resistance (T.D.R.) Transducer.

- The N.T.C. (Negative Temperature coefficient)Thermistor.

- Characteristics of an N.T.C. Thermistor (Resistance measuring method).

- Characteristics of the N.T.C. Thermistor used in an alarm circuit (double thermistor).

- Characteristics of a Type “K” Thermocouple.

Transducers for Light Measuring Applications:

- Characteristics of a photovoltaic cell.

- Characteristics of a photo-transistor.

- Luminous intensity detector.

- The P.I.N. Photodiode.

- Characteristics of a P.I.N. Photodiode. Linear Position transducers.

- Characteristics of a Linear Variable Differential Transformer (LVDT).

- Characteristics of a Variable Resistance.

- Characteristics of a Strain gauge Transducer.

Transducers for Environmental Measurement Applications:

- Characteristics of a air flow transducer.

- Characteristics of a pressure sensor.

- Characteristics of a humidity sensor.

Rotational Velocity Transducers or Position Measuring Applications:

- Characteristics of a slotted opto-transducers and its applications for counting and speed measurement.

- Characteristics of the reflective opto-transducers and Gray code disk.

- Characteristics of an inductive transducer.

- Characteristics of a Hall effect transducer.

- Characteristics of a D.C. permanent magnet Tachogenerator.

Transducers for Measuring Sound:

- Characteristics of a Dynamic Microphone.

- Characteristics of an ultrasonic receiver.

Transducers for Sound Output:

- Characteristics of the mobile coil loudspeaker.

- Characteristics of a Buzzer.

Output Transducer for Linear or Angular Movement:

- Characteristics of a D.C. Solenoid.

- Characteristics of a D.C. Relay.

- Characteristics of a Solenoid air valve.

- Characteristics of a Permanent Magnet Motor.

Signal Conditioning Circuits:

- Characteristics of the Direct Current amplifiers 1, 2 and x100.

- Characteristics of a current amplifier and application of a buffer amplifier.

- Characteristics of Power and Buffer Amplifiers.

- Characteristics of an Inverter Amplifier.

- Characteristics of a Differential Amplifier.

Signal Converter Circuits:

- Characteristics of a Voltage to Current Converter.

- Characteristics of a Current to Voltage Converter.

- Characteristics of a Voltage to Frequency Converter.

- Characteristics of a Frequency to Voltage Converter.

- Characteristics of a Full Wave Rectifier.

Comparators, Oscillator and Filters:

- Characteristics of a Comparator.

- Characteristics of an Alarm Oscillator circuit.

- Characteristics of an Electronic Switch.

- Characteristics of the Oscillator of 40 kHz.

- Characteristics of Filters.

Circuits that carry out Mathematical Operations:

- Characteristics of a Adding Amplifier.

- Characteristics of an Integrator.

- Characteristics of a Differentiator Circuit.

- Characteristics of a Sample and Hold Circuit.