Tender Specifications (for main items)

1 TA50/250C. Unit:

Bench-top wind tunnel in open circuit and with incompressible subsonic flow.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Inlet section with lips, a haven section and a 8:1 contraction ratio.

Transparent working area of 50 x 250 mm to assemble and test the different models.

The working area includes a Pitot static tube mounted in the top side.

Outlet section with diffuser that allows to avoid the generation of turbulences, which can generate damages the current quality in the working area.

Variable speed axial-flow fan, computer controlled:

Velocity range: 0 – 2800 rpm.

Air velocity in the working area: 0 – 27 m/s.

Sixty different pressure tappings (along the tunnel and in the models).

Thirty differential pressure sensors, range: 0 – 1 psi.

Several models and accessories are available, allowing a comprehensive study of subsonic aerodynamics.

All these models include several pressure tappings.

The complete unit includes as well:

Advanced Real-Time SCADA.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

National Instruments Data Acquisition board (250 KS/s, kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc. Apart from monitoring user's knowledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

Elements included:

- TA1. House Scale Model.

- TA2. Cylinder Model.

- TA3. Convex Semi-Cylinder Model.

Additional recommended elements (Not included):

- TA4. Car Model.

- TA5. Lorry Model.

- TA6. Lorry with Wind Deflector Model.

- TA7. Plane Model.

- TA8. Train Model.

- TA9. Projectile Model.

- TA10. Circular Disc Model.

- TA11. Wing of a Plane Model.

- TA12. Concave Semi-Cylinder Model.

- TA13. Blunt Element Model.

- TA14. Bernoulli Apparatus Model.

- TA15. Boundary Layer Plate Model.

- TA16. Model under “Custom Design”.

- TA17. Wake Survey Rake.

- TA18. Streamlined Shape (TA19 + TA50/250C-TARC models are required).

- TA19. Lift and Drag Balance (TA50/250C-TARC model is always required to operate TA18 and/or TA20 models).

- TA20. Drag Models (Sphere, Hemisphere, Convex, Hemisphere Concave, Circular Disc) (TA19 + TA50/250C-TARC models are required).

- TA50/250C-TARC. Forces Measurement Interface (TA19 model is always required to operate TA18 and/or TA20 models).

- TA50/250-BLE. Boundary Layer Experiment Accessory.

- TA50/250-SG1. Smoke Generator.

- APIV/30. PIV 2D flow Visualisation for Velocities up to 30 m/s and Area of 100 x 100 mm.

2 TA50/250C/CIB. Control Interface Box:

The Control Interface Box is part of the SCADA system.

Control interface box with process diagram in the front panel.

The unit control elements are permanently computer controlled.

Simultaneous visualization in the computer of all parameters involved in the process.

Calibration of all sensors involved in the process.

Real time curves representation about system responses.

All the actuators’ values can be changed at any time from the keyboard allowing the analysis about curves and responses of the whole process.

Shield and filtered signals to avoid external interferences.

Real time computer control with flexibility of modifications from the computer keyboard of the parameters, at any moment during the process.

Real time computer control for parameters involved in the process simultaneously.

Open control allowing modifications, at any moment and in real time, of parameters involved in the process simultaneously.

Three safety levels, one mechanical in the unit, another electronic in the control interface and the third one in the control software.

3 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

The Data Acquisition board model may change at any moment, providing the same or better features than those required for the unit.

4 TA50/250C/CCSOF. Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

5 Cables and Accessories, for normal operation.

6 Manuals:

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

*References 1 to 6 are the main items: TA50/250C + TA50/250C/CIB + DAB + TA50/250C/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

1.- Comprehensive study of subsonic aerodynamics and airflow studies.

2.- Measurement of pressure distribution around a two dimensional body.

3.- Study of flow visualization.

4.- Study of static pressure, dynamic pressure and total pressure using a Pitot' tube.

5.- Study of velocity measurement using a Pitot tube.

6.- Flux in a nozzle: Determination of the characteristics of the pressures field in a nozzle.

7.- Flux in a nozzle: Observation of the local characteristics, depending on whether the walls have a curvature or not, as well as what happens at the inlet and outlet areas of the contraction.

8.- House Scale Model (TA1): Study of aerodynamic forces due to the wind on a house.

9.- House Scale Model (TA1): Determination of the aerodynamic loads generated by the wind on a house walls.

10.- Cylinder Model (TA2): Study of flow around a cylinder.

11.- Cylinder Model (TA2): Determination of the form of the pressures field around a cylinder on which a current perpendicular to the axis impacts.

12.- Cylinder Model (TA2): Determination, by the type of detachment, whether the boundary layer finally becomes turbulent or remains laminar.

13.- Cylinder Model (TA2): Determination of the resistance coefficient of the cylinder.

14.- Cylinder Model (TA2): Relation of all the above-mentioned with the Reynolds's number.

15.- Convex Semi-Cylinder Model (TA3): Study of flow around a convex semi-cylinder.

16.- Convex Semi-Cylinder Model (TA3): Determination of the field of pressures in the convex semi-cylinder.

17.- Convex Semi-Cylinder Model (TA3): Determination of the aerodynamic resistance coefficients in the convex semi-cylinder.

Additional practical possibilities:

18.-Sensors calibration.

Additional practical possibilities to be done with the additional recommended elements (Not included):

19.- Study of aerodynamic forces due to the wind on a Car Model (TA4).

20.- Study of aerodynamic forces due to the wind on a Lorry Model (TA5).

21.- Study of aerodynamic forces due to the wind on a Lorry with Wind Deflector Model (TA6).

22.- Study of aerodynamic forces due to the wind on a Plane Model (TA7).

23.- Study of aerodynamic forces due to the wind on a Train Model (TA8).

24.- Study of aerodynamic forces due to the wind on a Projectile Model (TA9).

25.- Study of aerodynamic forces due to the wind on a Circular Disc Model (TA10).

26.- Study of aerodynamic forces due to the wind on a Wing of a Plane Model (TA11).

27.- Study of flow and pressure distribution around a Wing of a Plane Model (TA11) at different angles of attack.

28.- Study of aerodynamic forces due to the wind on a Concave Semi-Cylinder Model (TA12).

29.- Study of aerodynamic forces due to the wind on a Blunt Element Model (TA13).

30.- Study of effect of changing in Bernoulli Apparatus Model (TA14) the cross section and application of the Bernoulli equation.

31.- Study of laminar and turbulent boundary layer development with the Boundary Layer Plate Model (TA15) and the Boundary Layer Experiment Accessory (TA50/250-BLE).

32.- Study of static pressure, dynamic pressure and total pressure with the Wake Survey Rake (TA17).

33.- Study of aerodynamic forces due to the wind on a Streamlined Shape (TA18).

34.- Investigation into the influence of models shape in the drag forces (Lift and Drag Balance (TA19), Drag Models (TA20) and Forces Measurement Interface (TA50/250C-TARC)).

35.- Demonstration of flow patterns around different objects with the Smoke Generator (TA50/250-SG1).

36.- Study of the PIV 2D flow Visualisation for Velocities up to 30 m/s and Area of 100 x 100 mm (APIV/30).

Other possibilities to be done with this unit:

37.- Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

38.- Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivative parameters; etc, in real time.

39.- The Computer Control System with SCADA allows a real industrial simulation.

40.- This unit is totally safe as uses mechanical, electrical and electronic, and software safety devices.

41.- This unit can be used for doing applied research.

42.- This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

43.- Control of the TA50/250C unit process through the control interface box without the computer.

44.- Visualization of all the sensors values used in the TA50/250C unit process.

- Several other exercises can be done and designed by the user.

Tender Specifications (for optional items)

a) Technical and Vocational Education configuration

7 TA50/250C/ICAI. Interactive Computer Aided Instruction Software.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

8 TA50/250C/FSS. Faults Simulation System.

Faults Simulation System (FSS) is a Software package that simulates several faults in any EDIBON Computer Controlled Unit.

The "FAULTS" mode consists in causing several faults in the unit normal operation. The student must find them and solve them.

There are several kinds of faults that can be grouped in the following sections:

Faults affecting the sensors measurement:

- An incorrect calibration is applied to them.

- Non-linearity.

Faults affecting the actuators:

- Actuators channels interchange at any time during the program execution.

- Response reduction of an actuator.

Faults in the controls execution:

- Inversion of the performance in ON/OFF controls.

- Reduction or increase of the calculated total response.

- The action of some controls is annulled.

On/off faults:

- Several on/off faults can be included.

b) Multipost Expansions options

9 MINI ESN. EDIBON Mini Scada-Net System for being used with EDIBON Teaching Units.

EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The Mini ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- Mini ESN Software to control the whole system.

- Cables and accessories required for a normal operation.