TENDER SPECIFICATIONS

Measurements:

Carbon monoxide, CO (%).

Carbon dioxide, CO2 (%).

HC gasoline, propane, methane (ppm).

Oxygen, O2 (%).

Carbon monoxide adjusted (%).

Lambda factor.

Oil temperature.

Technical data:

Power supply: 115/230 VAC - 50/60 Hz - 68 W.

Measure time: 12 seconds (max.).

Warm-up time: 6 minutes (from 25° C).

Working temperature: from +2 °C to +40 °C.

Humidity: 95 % max. (non-condensing).

Pressure: 800 - 1060 mbar.

CO (+/- 0.02 % abs +/- 3 % rel): from 0 to 15 %.

CO2 (+/- 0.3 % abs +/- 3 % rel): from 0 to 20 %.

HCg (+/- 4 ppm abs +/- 3 % rel): from 0 to 15000 ppm vol.

HCp (+/- 8 ppm abs +/- 3 % rel): from 0 to 30000 ppm vol.

O2 (+/- 0.1 % abs +/- 5 % rel): from 0 to 25 %.

COc (+/- 1 % abs +/- 3 % rel): from 0 to 20 %.

Lambda: from 0 to 9.999 rmp.

Oil temperature: 0 - 200 °C.

Sample gas tube length: 6 m approx.

Sample gas tube diameter: 5 mm approx.

USB wire to computer.

Control software.

Additional recommended elements (Not included):

- TBMC3. Computer Controlled Test Bench for Single-Cylinder Engines, 2,2 kW.

- TBMC8. Computer Controlled Test Bench for Single-Cylinder Engines, 7.5 kW.

- TBMC12. Computer Controlled Test Bench for Single-Cylinder and Two-Cylinders Engines, 11 kW.

- TBMC75. Computer Controlled Test Bench for Four-Cylinders Engines, 75 kKw

- Dimensions: 600 x 200 x 300 mm approx.  (23.62 x 7.87 x 11.81 inches approx.). - Weight: 5 kg approx.  (11 pounds approx.).