EDIBON

Edition:ED01/25
Date:November/2025
TCEB
BOILING HEAT TRANSFER UNIT

TENDER SPECIFICATIONS
Bench-top unit.

Anodized aluminum frame and panels made of painted steel. 

Main metallic elements made of stainless steel. 

Diagram in the front panel with distribution of the elements similar to the real one.

Chamber: 

    Cylinder of boron silicate of high resistance.

    Internal diameter: 90 mm.

    External diameter: 100 mm. 

    Length: 300 mm.

Heating resistance of 690 W, stuffed in a copper cylinder of internal diameter: 12.7 mm, external diameter: 16 mm and length 50 mm.

Serpentine condenser: a copper tube plated with a surface of 0.043 m².

Load valve placed in the bottom part of the cylinder and it is used for charging and discharging of the unit.

Water flow control valve located in the conduction line of water. It regulates the water flow that inputs the serpentine.

Purge and safety valve.

Five temperature sensors ("J" type):

    Temperature sensor to measure the temperature of the hot surface.

    Temperature sensor to measure the temperature of the coolant.

    Temperature sensor at the water inlet.

    Temperature sensor at the water outlet.

    Temperature sensor to determine the temperature of the saturated vapour.

Pressure meter.

Wattmeter.

Flow meter.

Temperature switch, ready to cut off the electric current if the temperature exceeds the alarm value.

Pressure switch.

Designed to employ the coolant SES-36.

Electronic console:

    Metallic box.

    Heating resistance controller.

    Temperature sensors connections.

    Digital display for the temperature sensors.

    Selector for the temperature sensors.

    Wattmeter display.

    High pressure control connection.

Cables and accessories, for normal operation. 

Manuals: This unit is supplied whit the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

Unit:            

-Dimensions: 700 x 550 x 720 mm approx. (27.56 x 21.65 x 28.34 inches approx). -Weight: 50 kg approx. (110 pounds approx).  

Electronic console: -Dimensions: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx). -Weight: 10 kg approx. (22 pounds approx).

EXERCISES AND PRACTICAL POSSIBILITIES
1.-Visual demonstration of the three boiling modalities (convective, nucleate and film boiling).

2.-Determination of the thermal flow and the superficial heat transfer coefficient.

3.-Effect of the pressure on the critical thermal flow.

4.-Film condensation.

5.-Demonstration of the liquid dragging for the vapour.

6.-Relationship between the pressure and the temperature.

7.-Air effect in an installation.

Additional practical possibilities:

8.-Gauge pressure/Enthalpy.

9.-Properties of the SES-36.

Optional
TCEB/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features: 

• User Data Base Management. 

• Administration and assignment of Workgroup, Task and Training sessions. 

• Creation and Integration of Practical Exercises and Multimedia Resources. 

• Custom Design of Evaluation Methods. 

• Creation and assignment of Formulas & Equations. 

• Equation System Solver Engine. 

• Updatable Contents. 

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features: 

• Student Log-In & Self-Registration. 

• Existing Tasks checking & Monitoring. 

• Default contents & scheduled tasks available to be used from the first session. 

• Practical Exercises accomplishment by following the Manual provided by EDIBON. 

• Evaluation Methods to prove your knowledge and progression. 

• Test self-correction. 

• Calculations computing and plotting. 

• Equation System Solver Engine. 

• User Monitoring Learning & Printable Reports. 

• Multimedia-Supported auxiliary resources.
1

