TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum structure and panels in painted steel.

Main metallic elements in stainless steel.

Diagram in the front panel with similar distribution to the elements in the real unit.

Steam chamber: thick-walled glass cylinder with aluminum ends and PTFE seals:

Capacity: 0.5 – 1 kg of distilled water approx.

Inner diameter of the chamber: 95 mm.

Chamber length: 450 mm.

Two water cooled condensers, mounted in the upper cylinder cover:

Dimensions: 12.7 mm external diameter and 200 mm effective length.

They are specially designed and manufactured of cooper, incorporating a heat exchanger in order to reduce the surface temperature variation to a minimum:

Dropwise condenser gold plated.

Filmwise condenser natural finish.

Each condenser is provided with five connected temperature sensors (”K” type) to measure the mean metal temperature.

Electric heating element with thermal protection, power: 3 kW .

Air extraction system, composed by air cooler, separator and water jet vacuum pump with the necessary valves.

Pressure meter to measure the chamber pressure, range: -1 – 3 bar.

Two flow meters, range: 0.2 – 2  l/min.

Two temperature sensors (”K” type) to measure of steam at the top and bottom of the chamber.

Safety:

Pressure relief valve fitted to upper cylinder cover.

Safety pressure switch.

Heater thermal protection (120 ºC thermostat).

Electronic console:

Metallic box.

Temperature sensors connections.

Selector for temperature sensors.

Digital display for temperature sensors.

Switch and regulator for the electric heating element.

Wattmeter display.

On/Off main switch.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 700 x 570 x 770 mm approx. (27.55 x 22.44 x 30.31 inches approx.). Weight: 60 kg approx. (132.2 pounds approx.).

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 kg approx. (22 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Investigation of the saturation pressure/temperature relationship for H2O between about 20 ºC and 100 ºC.

2.- Visual demonstration of filmwise and dropwise condensation, and of nucleate boiling.

3.- Measurement of heat flow and surface heat transfer coefficient in both filmwise and dropwise condensation at pressures up to atmospheric.

4.- Demonstration and investigation of the effect of air in condensers.

5.- Demonstration of Dalton’s Law.

Optional

TCPGB/ICAI. Interactive computer aided instruction software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.