TENDER SPECIFICATIONS (for main items)

1 TDEGC/50 or TDEGC/60 Unit:

AEL-WBMP. Electrical Workbench (Mobile Small).

Control and Supervision Panel:

Enables to connect up to 16 diesel generators in parallel-island with distribution of active and reactive load and start/stop in function of the load demand.

Includes analogical outputs to control voltage and frequency regulators.

Three-phase measurements of the grid and generator voltage.

Three-phase measurements of the generator current and power.

Single-phase measurement of the grid current.

Sensor “island grid/parallel grid”.

Digital signal “local/remote control”.

Alarm indicators: over/undervoltage, reverse power, overcurrent, over/underfrequency, shutdown alarms and low level fuel.

Remote control signal states indicators: ready for operation, start request, GCB synchronization conditions, remote closing signal for GCB synchronization, mains parallel operation, isolated operation, emergency stop.

Maintenance panel: key of permission for maintenance, ignition switch, starter signal.

Back-Up protections: earth leakage relay, three-phase overcurrent relay.

Generator protections: max/mín-voltage (59/27), max/min-frequency (81O/U), dead bus detection, reverse power/reduce (32R/F), overcurrent time defined curve (50/51), inverse time overcurrent (IEC255).

Motor: over/sub speed (12).

PID Power Factor Control.

PID Excitation Control.

PID Active Power Control.

PID Speed Control.

Instrumentation Panel:

DC Power Analyzer:

Rated voltage range: 0 –100 VDC.

Rated current range: 0 – 10 A.

Communication port RS-485.

DC Analog Ammeter:

Rated current range: 0 – 2.5A.

AC Generator Power Analyzer:

Display for instantaneous variables: 3 x 3 digits.

Display for energies: 8 + 1 digits.

Voltage:

VL-N = 185 V to 460 V.

VL-L = 320 V to 800 V.

Current:

Phase current: 0.03 to 10 A.

Neutral current: 0.09 to 10 A.

AC Mains Power Analyzer:

Display for instantaneous variables: 3 x 3 digits.

Display for energies: 8 + 1digits.

Voltage:

VL-N = 185 V to 460 V.

VL-L = 320 V to 800 V.

Current:

Phase current: 0.03 to 10 A.

Neutral current: 0.09 to 10 A.

Frequency: 48 to 62 Hz  0.1 Hz.

Power: active, reactive and apparent.

Power factor: power factor for resistive, inductive and capacitive load types.

Connection Diagram and SCADA Panel:

Panel with real diagram.

Emergency stop push-button.

Three voltage indicators for three bus bar parts: generation indicator lamp, loads indicator lamp and mains indicator lamp.

Three synchronization lamps indicator.

GCB (generator circuit breaker) and MCB (mains circuit breaker) circuit breaker lamps.

P1 power connector for diesel engine-generator.

MAINS connector for synchronization operation.

USB 1 communication connector for maintenance operations.

ON / OFF switch.

230 VAC Power supply cable with single-phase plug.

230 VAC, 30mA differential protection.

Three-Phase Resistive Industrial Panel:

Three 1 kW three-phase resistor loads 3 x (3 x 150) ohms.

Three manual commutators to change the resistive load.

Three load indicator lamps.

Three-Phase Inductive Industrial Panel:

Three 1 kVar three-phase inductive loads

Three manual commutators to change the inductive load.

Three load indicator lamps.

Three-Phase Capacitive Industrial Panel:

Three 1 kVar three-phase capacitive loads.

Three manual commutators to change the capacitive load.

Three load indicator lamps.

Diesel Engine Generator Group:

Diesel Engine (The Diesel Engine may change at any moment, providing the same or better features than those required for the unit):

Four stroke diesel engine with cylinders in line.

Liquid cooled with axial fan.

Forced lubrication with vane pump on the crankshaft.

Water pump in the engine block.

Cylinder: 3.

Engine displ (cm3): 1028.

Injection system: indirect (IDI).

Compression ratio: 22.8:1.

Nominal power (kW / HP): 20 kW / 26,8 HP.

Max torque (Nm/rpm): 67.0 / 2000.

Diesel fuel.

Oil sump capacity (l): 2.4.

Phisical characteristics:

Dimensions: 519 x 516 x 412 mm.

Dry weight: 85 Kg.

Air filter.

Fuel tank: 160 l.

Synchronous generator:

Synchronous speed: 1500 rpm.

Nominal frequency: 50 Hz or 60 Hz (according to requested reference).

Apparent power: 8 kVA.

Nominal voltage: 400 VAC.

Nominal current: 11 A

Power factor: 0.8.

Number of poles: 4.

Nominal current excitation: 1.5 A.

Nominal voltage excitation: 23.7 A.

The complete unit includes as well:

Real and complete control of a diesel engine synchronous generator.

Automatic control over the engine speed and synchronous generator current excitation.

Diesel engine synchronous generator synchronization operations studies.

Diesel engine synchronous generator stand-alone operations studies.

Diesel engine-generator stability in stand-alone and parallel operations studies.

Studies of diesel engine synchronous generator in micro-grids, ships, black-out situations.

Analysis of engine PID speed control signal.

Analysis of synchronous generator PID excitation control signal.

Analysis of engine PID active power control signal.

Analysis of engine-synchronous generator PID power factor control signal.

Advanced Real-Time SCADA and PID Control.

Possibility of integration in Micro-Grids Power Systems (AEL-MGR).

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on Labview.

National Instruments Data Acquisition board (250 KS/s , kilo samples per second).

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

2 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

The Data Acquisition board model may change at any moment, providing the same or better features than those required for the unit.

3 TDEGC/CCSOF. PID Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Analog and digital PID control. PID menu and set point selection required in the whole work range.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

4 Cables and Accessories, for normal operation.

5 Manuals:

This unit is supplied with 8 manuals: Required services, Assembly and Installation, Control software, Starting-up, Safety, Maintenance, Calibration & Practices manuals.

*	References 1 to 5 are the main items: TDEGC/50 or TDEGC/60 Unit + DAB + TDEGC/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

1.- Automatic speed control of the engine and generator in island.

2.- Automatic excitation control of the three-phase synchronous generator.

3.- Automatic speed control of the engine in parallel with the grid.

4.- Automatic excitation control of the synchronous generator in parallel with the grid.

5.- Control of the engine and generator in parallel generation mode (working with other generators).

6.- Coordination of energy needs and generating capacity in stand-alone grids.

7.- Active power waves measurement in real time and comparison with PID speed signal and engine.

8.- Smart Meter to measure the generated energy by the generator.

9.- Automatic control of voltage and frequency of the generator/engine in a stand-alone or in parallel operation (with any other generator).

10.- Study of energy requirements and energy generation in stand-alone grid.

11.- Studying of synchronous generator/engine response when a  change in load is produced.

12.- Synchronization operations of the one synchronous generator and the grid.

Additional practical possibilities:

13.- Synchronization operations with two synchronous generators and grid (with any other generator).

14.- Load sharing between two synchronous generators (with any other generator).

Other possibilities to be done with this unit:

15.- Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

16.- Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivative parameters; etc, in real time.

17.- The Computer Control System with SCADA and PID Control allow a real industrial simulation.

18.- This unit is totally safe as uses mechanical, electrical/electronic, and software safety devices.

19.- This unit can be used for doing applied research.

20.- This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

21.- Visualization of all the sensors values used in the TDEGC unit process.

- Several other exercises can be done and designed by the user.

TENDER SPECIFICATIONS (for optional items)

a) Technical and Vocational Education configuration

6 TDEGC/ICAI. Interactive Computer Aided Instruction Software.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

b) Multipost Expansions options

7 MINI ESN. EDIBON Mini Scada-Net System for being used with EDIBON Teaching Units.

MINI ESN. EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The MINI ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA and PID Control integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA and PID Control, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- MINI ESN Software to control the whole system.

- Cables and accessories required for a normal operation.