TENDER SPECIFICATIONS

Anodized aluminum frame and panels made of painted steel.

The unit includes wheels to facilitate its mobility.

Diagram in the front panel with distribution of the elements similar to the real one.

The unit includes:

Three-way mixing valve with actuator motor:

Size: DN20.

Setting angle: 90°.

Opening time: 100 s.

A circulating pump:

Power consumption: 70 W.

Max. flow rate: 60  l/min.

Max. head: 5 m.

Four flowmeters:

Three flowmeters situated in the boiler water circuit (hot water), the return heating system water circuit (cold water) and the three-way mixing valve outlet, range: 2.5 – 28  l/min.

One flowmeter situated in the outlet of the differential pressure relief valve, range: 0.6 – 6.5  l/min.

Six bimetallic dial thermometers, range: 0 – 100 °C.

Five bleed valves (air vents) to remove the air from the water.

Two water connections with quick-release couplings for boiler water (hot water), size: DN15.

Two water connections with quick-release couplings for return heating system water (cold water), size: DN15.

Four ball valves allow control the boiler water flow and the return heating system water flow.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance and Practices manual.

Dimensions: 1900 x 550 x 1700 mm approx. (74.80 x 21.65 x 66.93 inches approx.).  Weight:100 Kg approx. (220 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Study of the function and operation of a three-way mixing valve.

2.-  Study of the function and operation of a differential pressure relief valve.

3.-  Study of the effect of mixing ratio on feed flow or circulating flow temperature.

4.-  Study of the effect of mixer setting on the flow rate.

Optional

TEV3V/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.