TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Hermetic compressor: power: 1 CV.

Two water condensers, capacity: 1580 W.

Two air condensers.

High pressure switch.

Storage vessel to prevent a wrong operation of the compressor, capacity: 0.5 l.

Filter for the working fluid.

Expansion valve.

Two water evaporators, capacity: 1580 W.

Two air evaporators.

Liquid separator to prevent liquid from entering the compressor, capacity: 0.73 l.

Four manometers, situated at important points of the process.

Thirteen “J” temperature sensor type.

Five flow meters:

Flow meters to measure the flow of coolant, range: 5 – 60  l/h.

Two water flow meters to measure the flow of water that circulates through the two water condensers, range: 0.2 – 2  l/min.

Two water flow meters to measure the flow of water that circulates through the two water evaporators, range: 0.2 – 2  l/min.

Compressor wattmeter.

Four-way valve (solenoid valve) for reversibility the cycle.

Four valves to direct the coolant through the condensers and evaporators.

Enthalpy diagram of R-513a.

This unit has been designed for the use with the refrigerant R-513a, CFC-free, environmental friendly.

Electronic console:

Metallic box.

Connectors for the temperature sensors.

Sensor selector for temperature sensors.

Digital display for temperature sensors.

High pressure control connection.

Fans switches. Fans speed regulators.

Cycle inversion valve connection. Cycle inversion valve switch.

Digital display for the wattmeter.

Compressor switch.

Main switch.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

Additional recommended elements (Not included):

- T/KIT1. Maintenance Kit containing Vacuum Pump, Hoses and Manometers.

- T/KIT2. Maintenance Kit containing Leakage Detector.

- T/KIT3. Maintenance Kit containing Refrigerant Filling and Evacuation Module.

Unit: - Dimensions: 1000 x 950 x 750 mm approx. (39.37 x 37.40 x 29.52 inches approx.) - Weight: 100 Kg approx. (220 pounds approx.).

Electronic console: - Dimensions: 490 x 450 x 470 mm approx. (19.29 x 17.71 x 18.5 inches approx.) - Weight: 20 Kg approx. (44 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Determination of COP (coefficient of performance) of a heat pump. Water as heat source. (Water-water heat pump).

2.- Determination of COP (coefficient of performance) of a heat pump. Air as heat source. (Water-air heat pump).

3.- Determination of COP (coefficient of performance) of a heat pump. Air as heat source. (Air-air heat pump).

4.- Determination of COP (coefficient of performance) of a heat pump. Water as heat source. (Air-water heat pump).

5.- Preparation of performance curves of the heat pump with different inlet and outlet temperatures. Water as heat source. (Water-water heat pump).

6.- Preparation of performance curves of the heat pump at different inlet and outlet temperatures. Air as a heat source. (Water-air heat pump).

7.- Preparation of performance curves of the heat pump with different inlet and outlet temperatures. Water as heat source. (Air-water heat pump).

8.- Preparation of the performance curves of the heat pump with different inlet and outlet temperatures. Air as heat source. (Air-air heat pump).

9.- Lay out of the steam compression cycle in a diagram P-H and comparison with the ideal cycle. Water as heat source. (Water-water heat pump).

10.- Lay out of the steam compression cycle in a diagram P-H and comparison with the ideal cycle. Air as heat source. (Water-air heat pump).

11.- Lay out of the steam compression cycle in a diagram P-H and comparison with the ideal cycle. Water as heat source. (Air-water heat pump).

12.- Lay out of the steam compression cycle in a diagram P-H and comparison with the ideal cycle. Air as heat source. (Air-air heat pump).

13.- Preparation of the performance curves of the heat pump based on the properties of the refrigerant and at different condensation and evaporation temperatures. Water as heat source. (Water-water heat pump).

14.- Preparation of the performance curves of the heat pump based on the properties of the refrigerant and at different condensation and evaporation temperatures. Air as heat source. (Water-air heat pump).

15.- Preparation of the performance curves of the heat pump based on the properties of the refrigerant and at different condensation and evaporation temperatures. Water as heat source. (Air-water heat pump).

16.- Preparation of the performance curves of the heat pump based on the properties of the refrigerant and at different condensation and evaporation temperatures. Air as heat source. (Air- air heat pump).

17.- Practices with cycle inversion.

Additional practical possibilities:

18.- Properties of the refrigerant R-513a.

19.- Enthalpy-Pressure diagram for the refrigerant R-513a.

Optional

THIBAR44B/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.