TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This unit is common for the required elements (at least one) (Not included).

This unit performs the following tasks:

Heating the water.

Pumping of hot water.

Change in the direction of cold water flows.

Cold and hot water measures.

Stainless steel tank (30  l), equipped with:

Electric heating element (3000 W) with thermostat (70 °C), to heat the water.

Temperature sensor (“J” type) to measure the water temperature.

Level switch to control the water level in the tank.

Stainless steel cover to avoid the contact with hot water. In this cover there is a hole that allows us to visualize the water level and also to stuff the tank.

Draining water valve.

Centrifugal pump, range: 0 – 3  l/min.

Two flow meters, one for hot water and the other for cold water, range: 0.2 – 2  l/min.

Control valves for the cold and hot water.

Four ball valves that, depending on how we manipulate them, give us co-current or counter-current flux in the exchanger.

Two ball valves to control and drain the hot water of the base unit.

Pressure regulator to avoid the introduction of too much pressure in the exchangers, tared at 0.6 bar.

Four flexible tubes to connect with the different exchangers.

Cables and accessories, for normal operation.

Electronic console:

Common for the required elements (at least one) (Not included).

Metallic box.

Temperature sensors connectors.

Digital display for the temperature sensors.

Selector for temperature sensors.

Level switch connector.

Heating element connector.

Heating element control (temperature control).

Fan switch.

Fan regulator.

Stirrer switch.

Pump connection.

Pump switch.

Cables and accessories, for normal operation.

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 1100 x 630 x 500 mm approx. (43.30 x 24.80 x 19.68 inches approx.). Weight: 50 Kg approx. (110.2 pounds approx.)

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 Kg approx. (22 pounds approx.)

Required elements (at least one) (Not included):

TITCB. Concentric Tube Heat Exchanger for TICB:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This Concentric Tube Heat Exchanger for TICB allows the study of heat transfer between hot water flowing through an internal tube and cold water flowing in the ring area lying between the internal and the external tubes.

This exchanger allows measuring hot and cold water temperatures at different points of the exchanger.

The exchanger is formed by two concentric copper tubes with hot water circulating through the interior tube and cold water circulating in the ring space.

This exchanger has two equal sections of 500 mm each one, where heat transfer takes place.

Exchange length: L = 2 x 0.5 = 1 m.

Internal tube:

Internal diameter: Dint = 16 • 10-3 m. External diameter: Dext = 18 • 10-3 m. Thickness = 10-3 m.

Heat transfer internal area: Ah = 0.0503 m². Heat transfer external area: Ac = 0.0565 m².

External tube:

Internal diameter: Dint = 26 • 10-3 m. External diameter: Dext = 28 • 10-3 m. Thickness = 10-3 m.

Six temperature sensors (“J” type):

Three temperature sensors for measuring cold water temperature:

Cold water inlet.

Cold water mid-position.

Cold water outlet.

Three temperature sensors for measuring hot water temperature:

Hot water inlet.

Hot water mid-position.

Hot water outlet.

Easy connection to the Base Service Unit, “TIUSB”.

Cables and accessories, for normal operation.

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 1100 x 630 x 320 mm approx. (43.30 x 24.80 x 12.60 inches approx.). Weight: 20 Kg approx. (44.09 pounds approx.)

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 Kg approx. (22 pounds approx.)

TITCAB. Extended Concentric Tube Heat Exchanger for TICB:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This Extended Concentric Tube Heat Exchanger allows the study of heat transfer between hot water flowing through an internal tube and cold water flowing in the ring area lying between the internal and the external tubes.

This exchanger allows measuring hot and cold water temperatures at different points of the exchanger.

“TITCAB” is a more sophisticated unit than “TITCB”, with four longer tube sections, giving four times the overall heat transfer area and three interim temperature measurement points (temperature sensors) in each fluid stream.

This exchanger has sufficient heat transfer area for demonstrating the typical counter-current flow conditions where the outlet of the heated stream is hotter than the outlet of the cooled stream.

The exchanger is formed by two concentric copper tubes with hot water circulating through the interior tube and cold water circulating in the ring space.

This exchanger has four equal sections of 1000 mm each one, where heat transfer takes place.

Exchange length: L = 4 x 1 = 4 m.

Internal tube:

Internal diameter: Dint = 16 • 10-3 m. External diameter: Dext = 18 • 10-3 m. Thickness = 10-3 m.

Heat transfer internal area: Ah = 0.0503 m². Heat transfer external area: Ac = 0.0565 m².

External tube:

Internal diameter: Dint = 26 • 10-3 m. External diameter: Dext = 28 • 10-3 m. Thickness = 10-3 m.

Ten temperature sensors (“J” type):

Five temperature sensors for measuring cold water temperature:

Cold water inlet.

Cold water at different interim positions (3).

Cold water outlet.

Five temperature sensors for measuring hot water temperature:

Hot water inlet.

Hot water at different interim positions (3).

Hot water outlet.

Easy connection to the Base Service Unit, “TIUSB”.

Cables and accessories, for normal operation.

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 1500 x 700 x 320 mm approx. (59.05 x 27.55 x 12.60 inches approx.). Weight: 25 Kg approx. (55.11 pounds approx.)

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 Kg approx. (22 pounds approx.)

TIPLB. Plate Heat Exchanger for TICB:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This Plate Heat Exchanger for TICB allows the study of heat transfer between hot and cold water through alternate channels formed between parallel plates.

The exchanger allows measuring cold and hot temperatures at the inlet and outlet of the exchanger.

Formed by corrugated stainless steel plates. It can be dismantled to observe its structure.

Four ports or connections of hot and cold water input and output.

Maximum flow: 12 m³/h.

Maximum work pressure: 10 bar.

Maximum work temperature: 100 °C.

Minimum work temperature: 0 °C.

Maximum number of plates: 20.

Internal circuit capacity: 0.176  l.

External circuit capacity: 0.22  l.

Area: 0.32 m².

Four temperature sensors (“J” type):

Two temperature sensors for measuring cold water temperature (inlet and outlet).

Two temperature sensors for measuring hot water temperature (inlet and outlet).

Easy connection to the Base Service Unit, “TIUSB”.

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 1100 x 630 x 320 mm approx. (43.30 x 24.80 x 12.60 inches approx.). Weight: 20 Kg approx. (44.09 pounds approx.)

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 Kg approx. (22 pounds approx.)

TIPLAB. Extended Plate Heat Exchanger for TICB:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This Extended Plate Heat Exchanger for TICB allows the study of heat transfer between hot and cold water through alternate channels formed between parallel plates.

The exchanger allows measuring cold and hot temperatures at different points of the exchanger.

Formed by corrugated stainless steel plates. It can be dismantled to observe its structure.

Four ports or connections of hot and cold water input and output.

Maximum flow: 12 m³/h.

Maximum work pressure: 10 bar.

Maximum work temperature: 100 °C.

Minimum work temperature: 0 °C.

Maximum number of plates: 20.

Internal circuit capacity: 0.176  l.

External circuit capacity: 0.22  l.

Area: 0.32 m².

Ten temperature sensors (”J” type):

Five temperature sensors for measuring cold water temperature (inlet, outlet and interim positions).

Five temperature sensors for measuring hot water temperature (inlet, outlet and interim positions).

Easy connection to the Base Service Unit, “TIUSB”.

Cables and accessories, for normal operation.

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions: Base Service Unit: 1200 x 700 x 320 mm approx. (47.24 x 27.55 x 12.60 inches approx.). Weight: 25 Kg approx. (55.11 pounds approx.)

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 Kg approx. (22 pounds approx.)

TICTB. Shell & Tube Heat Exchanger for TICB:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

The Shell & Tube Heat Exchanger for TICB consists on a group of tubes inside the heat exchanger. The hot water flows through the internal tubes and the cooling water circulates through the space between the internal tubes and the shell.

There are traverse baffles placed in the shell to guide the cold water to maximize the heat transfer.

Formed by tubes of stainless steel with hot water circulating in the interior.

Four segmented baffles located transversely in the shell.

Exchange length of the shell and each tube: L = 0.5 m

Interior tube (21 tubes):

Internal diameter: Dint = 8 • 10-3 m.

External diameter: Dext = 10 • 10-3 m.

Thickness = 10-3 m.

Internal heat transfer area: Ah = 0.0126 m².

External heat transfer area: Ac = 0.0157 m².

Shell:

Internal diameter: Dint,c = 0.148 m.

External diameter: Dext,c = 0.160 m.

Thickness = 6 • 10-3 m.

Seven temperature sensors (“J” type), for measuring cold and hot water temperatures at different points of the exchanger.

Easy connection to the Base Service Unit, “TIUSB”.

Cables and accessories, for normal operation.

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 1100 x 630 x 700 mm approx. (43.30 x 24.80 x 27.55 inches approx.). Weight: 30 Kg approx. (66.13 pounds approx.)

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 Kg approx. (22 pounds approx.)

TIVEB. Jacketed Vessel Heat Exchanger for TICB:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This Jacketed Vessel Heat Exchanger for TICB allows the study of heat transfer between hot water flowing through a jacket and the cold water contained in a vessel.

It can work in continuous supply or in a batch process (heating of a constant mass of water contained in a vessel).

The exchanger allows measuring temperatures at the inlet and outlet of the exchanger in cold as well as in hot water.

Inner tank:

Made of stainless steel.

Volume: 3.20  l.

Two overflows of different length that allow the exit of the water in the vessel through its upper part to maintain a constant flow during the process with a continuous supply.

Jacket surrounds the vessel through where hot water flows:

Made of stainless steel.

Jacket volume: 5.90  l.

Electric stirrer.

Five temperature sensors (“J” type):

Three temperature sensors for measuring cold water temperature.

Two temperature sensors for measuring hot water temperature.

Easy connection to the Base Service Unit, “TIUSB”.

Cables and accessories, for normal operation.

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 1100 x 630 x 700 mm approx. (43.30 x 24.80 x 27.55 inches approx.). Weight: 35 Kg approx. (77.16 pounds approx.)

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 Kg approx. (22 pounds approx.)

TIVSB. Coil Vessel Heat Exchanger for TICB:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This Coil Vessel Heat Exchanger for TICB allows the study of heat transfer between hot water flowing through a coil and cold water contained in the vessel.

It can work in continuous supply or in a batch process.

Formed by a methacrylate vessel, volume: 6  l.

Two overflows let the output of water from the vessel in the upper part in order to maintain the flow constant for continue supply process.

Copper coil where the water circulates:

Dint = 4.35 mm. Dext = 6.35 mm.

Total length of the tube that forms the coil: 1.5 m.

Electric stirrer.

Five temperature sensors (“J” type):

Three temperature sensors for measuring the cold water temperature.

Two temperature sensors for measuring the hot water temperature.

Easy connection to the Base Service Unit, “TIUSB”.

Cables and accessories, for normal operation.

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 1100 x 630 x 700 mm approx. (43.30 x 24.80 x 27.55 inches approx.). Weight: 30 Kg approx. (66.13 pounds approx.)

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 Kg approx. (22 pounds approx.)

TIFTB. Turbulent Flow Heat Exchanger for TICB:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This Turbulent Flow Heat Exchanger for TICB let us study the heat transfer between hot water that circulates through an internal tube and cold water that flows through the annular zone between the internal and the external tube. This exchanger let us measure cold water and hot water temperatures at different points of the exchanger.

Formed by two copper concentric tubes with hot water circulating through the internal tube and cold water circulating through the annular space.

The exchanger has four equal sections of 500 mm each one, where the heat transfer takes place.

Exchange length: L = 4 x 0.5 = 2 m.

Internal tube:

Internal diameter: Dint = 8 • 10-3 m.

External diameter: Dext = 10 • 10-3 m.

Thickness = 10-3 m.

Internal heat transfer area: Ah = 0.0377 m².

External heat transfer area: Ac = 0.0471 m².

External tube:

Internal diameter: Dint,c 13 • 10-3 m.

External diameter: Dext,c 15 • 10-3 m.

Thickness = 10-3 m.

Twelve temperature sensors (“J” type):

Cold water temperature sensor at the exchanger inlet or outlet.

Hot water sensor at the exchanger inlet.

Cold water sensor between the first and second stretch of the exchanger.

Hot water sensor between the first and second stretch of the exchanger.

Cold water sensor between the second and third stretch of the exchanger.

Hot water sensor between the second and third stretch of the exchanger.

Cold water sensor between the third and fourth stretch of the exchanger.

Hot water sensor between the third and fourth stretch of the exchanger.

Cold water temperature sensor at the exchanger inlet or outlet.

Hot water sensor at the exchanger outlet.

Temperature sensor of the exterior surface of the interior tube at the exchanger inlet.

Temperature sensor of the exterior surface of the interior tube at the exchanger outlet.

Easy connection to the Base Service Unit, “TIUSB”.

Cables and accessories, for normal operation.

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 1100 x 630 x 350 mm approx. (43.30 x 24.80 x 13.78 inches approx.). Weight: 20 Kg approx. (44.09 pounds approx.)

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 Kg approx. (22 pounds approx.)

TICFB. Cross Flow Heat Exchanger for TICB:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

The Cross Flow Heat Exchanger for TICB is designed to study heat transfer between two fluids in cross flow configuration.

A hot water flow coming from the base unit enters and leaves a radiator perpendicular to an air current, which is generated by a fan.

The heat exchanger allows to measure water and air temperatures at the inlet and outlet of the exchanger.

A Poly methyl methacrylate (PMMA) rectangular duct of 800 x 200 x 200 mm.

Radiator located across the air duct.

The fins of the radiator are made of aluminum and have a heat transfer area of 35000 mm².

Axial fan with speed regulation from the electronic console. It provides a maximum air velocity of 3 m/s.

Four “J” type temperature sensors to measure input and output water and air temperatures.

Anemometer to measure air velocity.

Two ball valves.

Easy connection to the Base Unit, “TIUSB”.

Cables and accessories, for normal operation.

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 1100 x 630 x 600 mm approx. (43.30 x 24.80 x 22.62 inches approx.). Weight: 20 Kg approx. (44.09 pounds approx.)

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 Kg approx. (22 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

Practices to be done with the Concentric Tube Heat Exchanger for TICB (TITCB):

1.-	Global energy balance in the heat exchanger and the study of losses.

2.-	Exchanger effectiveness determination. NTU Method.

3.-	Study of the heat transfer under counter-current and co-current flow conditions.

4.-	Flow influence on the heat transfer. Reynolds number calculation.

Practices to be done with the Extended Concentric Tube Heat Exchanger for TICB (TITCBA):

5.-	Global energy balance in the heat exchanger and the study of losses.

6.-	Exchanger effectiveness determination. NTU Method.

7.-	Study of the heat transfer under counter-current and co-current flow conditions.

8.-	Flow influence on the heat transfer. Reynolds number calculation.

Practices to be done with the Plate Heat Exchanger for TICB (TIPLB):

9.-	Global energy balance in the heat exchanger and the study of losses.

10.-Exchanger effectiveness determination. NTU Method.

11.-Study of the heat transfer under counter-current and co-current flow conditions.

12.-Flow influence on the heat transfer. Reynolds number calculation.

Practices to be done with the Extended Plate Heat Exchanger for TICB (TIPLAB):

13.-Global energy balance in the heat exchanger and the study of losses.

14.-Exchanger effectiveness determination. NTU Method.

15.-Study of the heat transfer under counter-current and co-current flow conditions.

16.-Flow influence on the heat transfer. Reynolds number calculation.

Practices to be done with the Shell & Tube Heat Exchanger for TICB (TICTB):

17.-Global energy balance in the heat exchanger and the study of losses.

18.-Exchanger effectiveness determination. NTU Method.

19.-Study of the heat transfer under counter-current and co-current flow conditions.

20.-Flow influence on the heat transfer. Reynolds number calculation.

Practices to be done with the Jacketed Vessel Heat Exchanger for TICB (TIVEB):

21.-Global balance of energy in the heat exchanger and losses study.

22.-Determination of the exchanger effectiveness. NTU Method.

23.-Influence of the flow on the heat transfer. Calculation of the number of Reynolds.

24.-Influence of the vessel stirring on the heat transfer when operating in batches.

25.-Influence of the vessel's water volume on the heat transfer when operating in batches.

Practices to be done with the Coil Vessel Heat Exchanger for TICB (TIVSB):

26.-Global balance of energy in the heat exchanger and the study of losses.

27.-Determination of the exchanger effectiveness. NTU Method.

28.-Influence of the flow on the heating transfer. Calculation of Reynolds number.

29.-Influence of the stirring vessel on the heat transfer with operation in batches.

30.-Influence of the water volume in the vessel on the heat transfer with operation in batches.

Practices to be done with the Turbulent Flow Heat Exchanger for TICB (TIFTB):

31.-Global energy balance in the heat exchanger and losses study.

32.-Determination of the exchanger effectiveness. NTU Method.

33.-Study of the heat transfer in counter-current and co-current flow conditions.

34.-Flow influence on heat transfer. Reynolds number calculation.

35.-Obtaining of the correlation that relates Nusselt number with Reynolds number and Prandtl number.

36.-Obtaining of the heat transfer coefficients by convection.

Practices to be done with the Cross Flow Heat Exchanger for TICB (TICFB):

37.-Introduction to the concept of psychometric properties.

38.-Effect of temperature differential on the heat transfer coefficient.

39.-Familiarization with cross flow heat exchanger.

40.-Overall energy balance in the heat exchanger and study of losses.

41.-Determination of the exchanger effectiveness (NTU method).

42.-Influence of air and water flow on the heat transfer. Reynolds number calculation.

Optional

TIUSB/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.