TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This Coil Vessel Heat Exchanger for TICB allows the study of heat transfer between hot water flowing through a coil and cold water contained in the vessel.

It can work in continuous supply or in a batch process.

Formed by a methacrylate vessel, volume: 6  l.

Two overflows let the output of water from the vessel in the upper part allowing two work volumes to be adjusted.

Copper coil where the water circulates:

Dint = 4.35 mm.

Dext = 6.35 mm.

Total length of the tube that forms the coil: 1.5 m.

Electric stirrer.

Five temperature sensors (“J” type):

Three temperature sensors for measuring the cold water temperature.

Two temperature sensors for measuring the hot water temperature.

Easy connection to the Base Service Unit, “TIUSB”.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Required elements (Not included):

- TIUSB. Base Service Unit (Common for all available Heat Exchangers type "TI..B").

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

This unit performs the following tasks:

Heating the water.

Pumping of hot water.

Change in the direction of cold water flows.

Cold and hot water measures.

Stainless steel tank (30  l), equipped with:

Electric heating element (3000 W) with thermostat (70 °C), to heat the water.

Temperature sensor (“J” type) to measure the water temperature.

Level switch to control the water level in the tank.

Stainless steel cover to avoid the contact with hot water. In this cover there is a hole that allows us to visualize the water level and also to stuff the tank.

Draining water valve.

Centrifugal pump, range: 0 – 3  l/min.

Two flow meters, one for hot water and the other for cold water, range: 0.2 – 2  l/min.

Control valves for the cold and hot water.

Four ball valves that, depending on how we manipulate them, give us co-current or counter-current flux in the exchanger.

Two ball valves to control and drain the hot water of the base unit.

Pressure regulator to avoid the introduction of too much pressure in the exchangers, tared at 0.6 bar.

Four flexible tubes to connect with the different exchangers.

Electronic console:

Metallic box.

Temperature sensors connectors.

Digital display for the temperature sensors.

Selector for temperature sensors.

Level switch connector.

Heating element connector.

Heating element control (temperature control).

Fan switch.

Fan regulator.

Stirrer switch.

Pump connection.

Pump switch.

Dimensions: Unit: 1100 x 630 x 700 mm approx. (43.30 x 24.80 x 27.55 inches approx.). Weight: 30 kg approx. (66.13 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Global balance of energy in the heat exchanger and the study of losses.

2.- Determination of the exchanger effectiveness. NTU Method.

3.- Influence of the flow on the heating transfer. Calculation of Reynolds number.

4.- Influence of the stirring vessel on the heat transfer with operation in batches.

5.- Influence of the water volume in the vessel on the heat transfer with operation in batches.

Optional

TIVSB/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.