TENDER SPECIFICATIONS

Bench-top unit to demonstrate how temperature measurements can be influenced by sources of thermal radiation.

The objective of this module is to measure the error in a black thermocouple due the radiation with respect with another normal thermocouple where there are not radiative shielding in comparison when there are radiative shielding, error in function of material of the thermocouple’s capsule, size of the thermocouple, etc.

Anodized aluminum frame and panels made of painted steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Variable speed fan, which generates air flux through the duct, range: 0-1200 l/min, regulated from the electronic console (0-100%).

Stainless steel duct with interior cover, including:

Temperature sensor, “T” type (high precision), in order to measure the temperature of inlet air.

Flow sensor for measuring the air flow generated in the duct.

Temperature sensor, “T” type (high precision), in order to measure the temperature of outlet air.

Copper cylinder with exterior cover:

Interior heating element.

Temperature sensor, for measuring the temperature of the cylinder.

Five temperature sensors, “T” type (high precision), with different styles and sizes installed in the duct to demonstrate the differences in readings obtained:

Temperature sensor of bare.

Temperature sensor of inconel.

Temperature sensor of s/steel.

Temperature sensor of black s/steel.

Temperature sensor of ceramic.

The power of the heating element is regulated from the electronic console (0-100%).

Power measurement (Wattmeter) from the electronic console.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions: 430 x 350 x 1300 mm approx. (16.93 x 13.78 x 51.18 inches approx). Weight: 50 Kg approx. (110 pounds approx).

Required elements (Not included):

- TSTCB/EC. Electronic Console (Common for available Modules type “TXC/...B”):

Metallic box.

Temperature sensors connections.

Digital display for temperature sensors.

Selector for temperature sensors.

Level switch connection.

Air flow sensor connection.

Digital display for the air flow sensor.

Radiation sensor connection.

Digital display for the thermal radiation.

Switch and regulator for the heating element power (AR-1) (0-100%).

Heating element (AR-2) controller. It is a programmer to fix a setpoint and prevent the heating element temperature from exceeding that setpoint. The maximum safety temperature can never be exceeded.

Switch and regulator for the fan flow rate (0-100%).

Pump switch.

Digital display (wattmeter) for the power of the resistance AR-1, it is shown in watts.

Main switch.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

EXERCISES AND PRACTICAL POSSIBILITIES

1.-Radiation errors in temperature measurement.

2.-Measurement the errors in thermocouples in function of the painting, material of its capsule, size.

3.-Effect of air velocity on measurement error.

Optional

TXC/ERB/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.