TENDER SPECIFICATIONS

Bench-top unit to study the principles of linear heat conduction and to allow the conductivity of various solid conductors and insulators to be measured.

It is given with interchangeable samples of different materials, different diameters and different insulating materials that allow to demonstrate the area effects, the conductivity and the combinations in series in the heat transfer process.

Anodized aluminum frame and panels made of painted steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Input heat section.

Electric heater.

Refrigeration section with a surface cooled by water.

Interchangeable central sections:

With brass of 25 mm of diameter.

With brass of 10 mm of diameter.

With stainless steel of 25 mm of diameter.

Water flow regulation valve.

Thermal paste is supplied to demonstrate the difference between poor and good thermal contact between the sections.

Nineteen temperature sensors, “T” type (high precision):

Seventeen temperature sensors distributed in the heating section (4 sensors), refrigeration section (4 sensors) and central sections (3 sensors in each central section).

Temperature sensor at the water inlet of the unit.

Temperature sensor at the water outlet of the unit.

The power of the heating element is regulated from the electronic console (0-100%).

Power measurement (Wattmeter) from the electronic console.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting up, Safety, Maintenance & Practices Manuals.

Dimensions:  400 x 300 x 300 mm approx. (15.74 x 11.81 x 11.81 inches approx). Weight: 20 Kg approx. (44 pounds approx).

Required elements (Not included):

- TSTCB/EC. Electronic Console (Common for available Modules type “TXC/...B”):

Metallic box.

Temperature sensors connections.

Digital display for temperature sensors.

Selector for temperature sensors.

Level switch connection.

Air flow sensor connection.

Digital display for the air flow sensor.

Radiation sensor connection.

Digital display for the thermal radiation.

Switch and regulator for the heating element power (AR-1) (0-100%).

Heating element (AR-2) controller. It is a programmer to fix a setpoint and prevent the heating element temperature from exceeding that setpoint. The maximum safety temperature can never be exceeded.

Switch and regulator for the fan flow rate (0-100%).

Pump switch.

Digital display (wattmeter) for the power of the resistance AR-1, it is shown in watts.

Main switch.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Obtaining of the curve of thermal conductivity of the air.

2.- Thermal conductivity in vacuum.

3.- Water thermal conductivity determination.

4.- Thermal conductivity determination of a mineral oil.

5.- Dry air thermal conductivity under atmospheric pressure.

Optional

TXC/LGB/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.