TENDER SPECIFICATIONS

Flexible charging hose set in three colors: red, blue and yellow for emptying and refrigerant charging:

Female connections for 1/4” SAE.

Length: 1.5 m.

Reinforced synthetic rubber with high resistance to refrigerants and low permeability.

Easy to thread brass terminals.

Spark-free, two-stage vacuum pump with solenoid valve and vacuum gauge:

Flow rate: 70 LPM.

Power: 1/3 HP.

Vacuum capacity: 0.02 mbar.

Connection: 1/4” SAE.

Piston / Bridge-pressure and temperature gauge set, for air conditioning and two-way refrigeration systems. With scale for refrigerants: R-22, R-404a and R-134a:

Adjustable manometer group with 60 mm diameter.

Sight glass to see the refrigerant gas.

Quick controls, easy to use.

Easy to read.

Equipped with a clamping hanger.