TENDER SPECIFICATIONS

Reclaimer to transfer the refrigerant gas from the cooling system to the appropriate cylinder without applying any process to the refrigerant:

Recovery capacity for all CFC, HCFC and HFC and HFO refrigerant gas:

Steam up to 16 kg/h.

Liquid up to 111 kg/h.

Push-Pull up to 373 kg/h.

Dry piston compressor.

Oil distillation system.

Automatic safety lock: 38.5 bar.

Visual signalling (green LED) of end of recovery.

Flexible charging hose set in three colors: red, blue and yellow for emptying and refrigerant charging:

Female connections for 1/4” SAE.

Length: 1.5 m.

Reinforced synthetic rubber with high resistance to refrigerants and low permeability.

Easy to thread brass terminals.

- Dimensions: 550 x 300 x 400 mm approx. (21.65 x 11.81 x 15.74 inches approx.) - Weight: 20 kg approx. (44 pounds approx.)