TENDER SPECIFICATIONS

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Sieve Plates Column with eight plates with one temperature taking and sample:

Internal diameter: 50 mm.

Length: 1000 mm.

Vacuumed, silverplated and double transparent band for vision.

Column head with temperature taking and conical output for distilled product.

Column head with a valve for the steam distribution.

Boiler (with sample outputs) with heating mantle, capacity: 2  l.

Distillation collector of graduated glass, capacity: 2  l.

Refrigerator.

Temperature measurement system.

Seven ”J” type temperature sensors.

Working temperature: ambient temperature up to 125 °C.

Flow meter, range: 0.2 – 2  l/min.

Manometer tubes.

Solenoid valve.

Electronic console:

Metallic box.

Temperature sensors connections.

Digital display for temperature sensors.

Selector for temperature sensors.

Heating element temperature control.

Solenoid valve switch.

Solenoid valve timer controller (Solenoid valve reflux time control).

Main switch.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Additional recommended elements (Not included):

- CAR1. Raschig Ring Column.

- C8P1. 8 Plates Type Column (1 Temperature Point).

- C10P10. 10 Plates Type Column (10 Temperature Points).

- C14P14. 14 Plates Type Column (14 Temperature Points).

Dimensions: Unit: 900 x 600 x 2600 mm approx. (35.43 x 23.62 x 102.36 inches approx.). Weight: 170 Kg approx. (375 pounds approx.)

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 12 Kg approx. (26.45 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Preparation of solutions.

2 .- Analytic valuation techniques.

3 .- Filling of the column.

4 .- Batch operation.

5 .- Obtaining the McCabe-Thiele diagram. Without reflux.

6 .- Obtaining the number of plates. Without reflux.

7.-  Efficiency calculations. Without reflux.

8.-  Variation of the composition of the distilled product: constant reflux ratio.

9.-  Constant composition of the distilled product: variation of reflux ratio.

10.-Constant composition of the distilled product: constant reflux ratio.

11.-Mass and energy balances across the system.

12.-Plates fluid dynamics studies, including load loss and column flooding.

13.-Calculation of the theoretical number of floors in the plates columns,and the equivalent height of the theoretical floor (HEPT) in the filling columns (Raschig rings).

14.-Pursuit of the temperatures in all plates in the column (Plates columns).

15.-Study of the rectification efficiency.

16.-Demonstration of azeotropic distillation.

17.-Studies of heating interchange in glass refrigerators.

Optional

UDDB/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.