TENDER SPECIFICATIONS

Anodized aluminum frame and panels made of painted steel.

The unit includes wheels to facilitate its mobility.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Feed liquid vessel, capacity: 9  l.

Product liquid vessel, capacity: 9  l.

Feed hopper with feed endless screw for solids.

Motor for feed endless screw.

Main rotary extraction vessel with eight cells of extraction.

Motor for the rotation of the main extraction vessel.

Variable rotation speed.

Three sprinklers.

Solid products exit.

Conductivity meter, and four conductivity cells:

Three conductivity scales:

200 S  0.1 – 199.9 S.

2 mS  0.2 – 1.999 mS.

20 mS  2 – 19.99 mS.

* S: microSiemens; mS: miliSiemens.

Four temperature sensors, ”J“ type.

Three heating resistances, range: 360 W each one.

Three safety thermostats: 70 ºC.

Three peristaltic pumps, with speed regulator: 12.7 l/h.

Three decanting filters (in-line strainer).

Circulation valves to change the circuit configuration.

Solvent temperatures: ambient to 50 ºC.

Electronic console:

Temperature sensors connections.

Digital display for temperature sensors.

Selector for temperature sensors.

Three controllers for the three heating elements.

Pumps speed controller.

Touch screen for controlling:

Number of revolutions of the feed motor.

Number of cycles.

Cycle time.

Main switch.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 705 x 570 x 1680 mm approx. (27.75 x 22.44 x 66.14 inches approx.). Weight: 100 Kg approx. (220 pounds approx.).

Electronic console: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.). Weight: 10 Kg approx. (22 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Demonstration of the operation of a continuous multiple stage process.

2.-  Closed circuit percolation extraction (batch operation).

3.-  Open loop percolation extraction (continuous operation).

4.-  Investigation of one, two and three stage continuous processes.

5.-  Investigation into effect of solvent temperatures.

6.-  Investigation into effect of solvent flow rates.

7 .- Investigation into effect of processing time.

8.-  Process economics. Process efficiency.

9.-  Mass balances.

10.-Influence of the particle size.

11.-Influence of the stages numbers.

12.-Influence of the solvent type.

13.-Extractions of inorganic and aqueous components.

14.-Test of extractions for industrial use.

Optional

UESLB/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.