TENDER SPECIFICATIONS

The “WIT-PGH” application includes the following elements:

• FP-STR. Assembly Frame with Safe Electrical Power Supply.

Aluminum frame:

Two aluminum struts.

Easy assembly of components via hammer head screws.

Possibility of simultaneous work of several students.

Four swiveling casters to facilitate the movement.

Dimensions:

Frame height: 1800 mm.

Useful working height: 1000 mm.

Width: 1500 mm.

Three-phase connection plug.

Safe electric box:

Differential magnetothermal, 4 poles, 25 A, 30 mA AC 6 KA.

Emergency stop mushroom (230/400 VAC).

5-wire hose for connection to frame.

Signal lamp of voltage presence.

• FP-KIT-2. DC Motor Controller Installation Kit.

DC Motor controller:

Supply voltage: 230 VAC (PH + N).

Three ON/OFF switches:

ON/OFF power switch.

Star-stop control activation switch.

Internal tachodynamo feedback switch.

Two potentiometers:

Speed control.

Torque control.

Terminals:

Rotor terminals.

Excitation terminals.

Tachodynamo terminals.

Three driver status LED (red, yellow and green).

Variable output voltage: 0 - 300 VDC.

Electric cabinet with door:

Dimensions:

Height: 500 mm.

Width: 400 mm.

Depth: 200 mm.

Cut rail 2 m bar.

Grey cable duct bar of 2 m for wiring.

200 W DC motor with wound rotor.

• FP-KIT-4. Wiring Installation Kit.

100 m of grey wire of 1.5 mm².

100 m of brown wire of 1.5 mm².

100 m of black wire of 1.5 mm².

100 m of green/yellow wire of de 1.5 mm².

25 m of screened wire.

• FP-KIT-10. Tubes and Fittings for Channeling Wiring Kit.

PVC electrical conduit for wiring installation, 20 m.

7 x Electrical box.

Cable guide.

20 x Cable clamp, 20 mm.

• FP-KIT-10/B. Rigid Tube Kit.

20 m PVC rigid tube.

10 x Threaded connector.

5 x Elbow for conducts clamping.

• FP-KIT-16. Photovoltaic Generation Kit.

Ammeter analog DC 0 - 10 A.

Current cut-off switch.

Photovoltaic charge regulator.

Two lamps panel to simulate solar radiation:

Power: 2 x 250 W.

Switch ON/OFF.

Current regulator.

Aluminum frame.

96 W photovoltaic panel:

Maximum power: 96 W.

Voltage at maximum power: 17.8 V.

Solar charge controller.

10 A DC ammeter.

• FP-KIT-19. Lighting Consumption Kit.

Four sockets.

Two 12 VDC lamps.

Two halogen lamps 230 VAC (PH + N).

Four switches.

• FP-KIT-22. Hybrid Installation Kit.

DC analog ammmeter from 0 - 30 A.

Current cutoff switch.

Sine-wave inverter:

Nominal power: 1000 VA.

Input voltage: 12 VDC.

Output voltage: 230 VAC.

Battery charger.

Battery:

Battery voltage: 12 VDC.

Battery capacity: 90 Ah.

• FP-KIT-23. Wind Turbine Installation for Hybrid Systems Kit.

DC analog ammeter from 0 - 10 A.

Dump load implemented in the controller.

Switch to stop generator spinning.

Permanent magnet wind turbine:

Power output: 200 W.

Nominal wind speed: 11.5 m/s.

Charge regulator:

Output voltage: 12 VDC.

• FP-KIT-52. Meters, Switches and Distribution Busbar Installation Kit.

Distribution electric box.

Differential thermal switch, two poles, 16 A.

2 x Energy power meters.

Distribution busbar:

It consists of two rails:

One for the connection of positive poles.

One for the connection of negative poles.

Required elements (Not included):

• CHER. Tool Box.

Crimper.

Tin.

Meter.

Insulating tape.

Heat shrink.

Pin punch.

Star and flat screwdrivers.

Allen keys.

Soldering iron and soldering iron support.

Rubber hammer.

Connection terminals.

Cable ties.

Cable cutters.

Clamps.

Additional recommended elements (Not included):

• MED65. Digital Multimeter.

This module has a digital multimeter of about 3 ½ digits, with double-jack ending cables of about 4 mm to facilitate interconnections.

With this digital multimeter we will be able to measure:

Voltage.

Current.

Resistance.

Capacitors capacity.

Temperature.

• FP-KIT-5. Measuring Kit.

Clamp meter:

Clamp for alternating/direct current measurements contactless.

The clamp can measure:

Current.

Voltage.

Resistance.

A voltage and continuity tester:

Voltage range: 12 - 690 VAC.

Phases rotating detection in three-phase systems.

Polarity tester.

• FP-GENS. Genset.

Single-phase.

Nominal voltage: 230 VAC.

Maximum power: 1000 W.

Frequency: 50 Hz.

Approx autonomy: 6 h.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 1500 x 400 x 1800 mm approx. (59.05 x 15.74 x 70.86 inches approx.) - Weight: 80 Kg approx. (176 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Wiring of the needed protections for the island operation of solar and wind systems.

2.- Wiring of the photovoltaic panel to the solar load regulator.

3.- Wiring of the solar load regulator to the busbar.

4.- Wiring of the busbar to the battery.

5.- Wiring of the battery to the inverter.

6.- Simulation of different solar radiations.

7.- Start-up of the photovoltaic generation system.

8.- Voltage and current measurement at the outlet of the photovoltaic panel and at the battery.

9.- Wiring of the wind turbine to the wind load controller.

10.- Wiring of the wind load regulator to the busbar.

11.- Simulation of different wind conditions.

12.- Start-up of the wind generation system.

13.- Voltage and current measurement at the outlet of the wind turbine and at the battery.

14.- Start-up of the hybrid generation system.

15.- Wiring of the generator group to the current inverter.

16.- Start-up of the auxiliary generating group.

17.- Wiring of light loads.

- Several other exercises can be done and designed by the user.

Optional

WIT-PGH/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.