TENDER SPECIFICATIONS

• N-ALI02. Domestic Main Power Supply Module.

Supply voltage (Single-Phase): 230 VAC, PH+N+G.

ON-OFF removable key.

Output Voltage Connections:

Two Single-Phase: 230 VAC.

Single-Phase supply hose connecting plug.

Differential magnetothermal, 2 poles, 25A, 30mA AC 6KA.

• N-ALI03. AC Auxiliary Power Supply (24 Vac) Module.

Supply voltage (Single-Phase): 230VAC PH+N+G.

Output voltage:

Single-Phase 24 VAC/12 VAC.

24 VDC.

0-24 VDC through potentiometer.

• N-ZCC01. Z-WAVE+ZigBee Control Central Module 1.

Wireless access to Z-WAVE networks.

Create custom scenes and sequences.

Compatibility with ZigBee protocol.

Control of a large number of devices connected at the same time.

• N-ZCT01. Z-WAVE+ZigBee Controller with Touch Screen 1.

Wireless access to Z-WAVE networks.

Create custom scenes and sequences.

Compatibility with ZigBee protocol.

Large number of integrated sensors compatible with most Z-WAVE devices.

Automation of device control options.

Touch screen.

• N-ZMTB. Z-WAVE Motion, Temperature and Brightness Sensor Module.

Wireless Technology: Z-WAVE Protocol.

Wireless Motion Detector.

Detects movements with a passive infrared sensor.

Temperature measurement.

Light brightness measurement.

• N-ZPCM. Z-WAVE Plug with Consumption Measurement Module.

Wireless Technology: Z-WAVE Protocol.

Integrates directly into an electrical plug.

Measurement of power consumption of the connected load.

Local load control.

Instantaneous consumption and total consumption values are sent to the Z-WAVE Control Unit.

• N-ZRPC Z-WAVE Radio Remote Control with 5 push-buttons.

Wireless Technology: Z-WAVE Protocol.

Control via the basic ON/OFF command.

Control devices remotely by connecting this remote control to the Z-WAVE network.

• N-ZSS. Z-WAVE Surface Switch Module.

Wireless Technology: Z-WAVE Protocol.

Options as main domotic controller or as secondary Z-WAVE controller.

• N-ZPB. Push-Button Module.

Wireless Technology: Z-WAVE Protocol.

Wireless control button.

It allows to control the devices of the Z-WAVE network and to carry out several scenes that take place in the domotic system.

• N-ZILD. Z-WAVE Incandescent Lighting Dimmer  Control Module.

Wireless Technology: Z-WAVE Protocol.

Supports control of low-voltage halogen lamps with electronic transformer, dimmable compact fluorescent lights and dimmable LED bulbs.

It allows the creation of complex scenarios.

• N-LAM16. Halogen Lamp.

Nominal voltage: 230 VAC (PH+N).

Halogen lamp.

Power: 60 W.

• N-ZVL. Z-WAVE Variable Led Lamp Module.

Wireless Technology: Z-WAVE Protocol.

LED white light bulb that can be controlled remotely.

Adjustable brightness.

• N-REF. Single-Phase Fixed Resistor.

Value: 150 ohm.

Maximum power: 500 W.

Selector:

Position 0:circuit opened.

Position 1: circuit closed.

Current fuse: 2 A.

• AEL-PC. Touch Screen and Computer.

Touch Screen:

Energy efficiency class: A.

Screen diagonal: 68.6 cm (27 inch (s)).

Power consumption (operating): 26 watts.

Annual energy consumption: 38 kWh.

Power consumption (standby / off) 0.49 watts.

Screen resolution: 1920 x 1080 pixels.

Computer:

Processor Number: Intel Core i7-6600U Processor (4M Cache, up to 3,40 GHz).

Cache: 4 MB Intel Smart Cache.

Clock speed: 2.6 GHz.

# Of Cores/# of Threads: 2/4.

Max. TDP/Power: 15 W.

Memory Types: DDR4-2133, LPDDDR3-1866, DDR3L-1600.

Graphics: Intel HD Graphics 530.Slot for PCI Express.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

ZWAVE-UB:

- Dimensions: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.)

- Weight:       10 Kg approx. (22 pounds approx.)

ZWAVE-LC:

- Dimensions: 490 x 330 x 310 mm approx.  (19.29 x 12.99 x 12.20 inches approx.)

- Weight:       10 Kg approx. (22 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

Some practical exercises with Lighting Control “ZWAVE-LC”:

1.-  Setting of the motion, temperature and brightness multi sensor module.

2.-  Setting of the plug with energy measurement module.

3.-  Setting of the smoke detection module.

4.-  Setting of the 5 push-button remote controller.

5.-  Setting of the surface commutation module.

6.-  Setting of the dimmer module for illumination control.

7.-  Setting of the led lamp module for illumination control.

8.-  Star-up and registration of the control central, connecting the tablet or smartphone (not included), programming of each device based on rules and preset scenes.

- Several other exercises can be done and designed by the user.

Optional

AEL-ZWAVE/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.